DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #2

Activity Name: 	 Switch Interfacing

| Name of Student: | Agneay B Nair |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24102 |
| Activity Date: | 18-08-2026 |
| Submission Date: | 18-08-2026 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to interface switches. Debug and verify the program using IDE on a development computer system.

CODE:

#include<LPC213x.h>

void delay(int n){

int i;

for(i = 0;i<n*1000;i++);

}

int main(){

PINSEL0 = 0x00000000;

IODIR0 |= (1<<16);

IODIR0 &= ~(1<<8);

while(1){

if(IOPIN0 & (1<<8)){

// turn LED on

IOSET0 = (1<<16);

delay(10);

}else{

IOCLR0 = (1<<16);

delay(10);

}

}

}

OUTPUT: