DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #8

Activity Name: Real-Time Clock Interfacing

| Name of Student: | Agneay B Nair |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24102 |
| Activity Date: | 21-08-26 |
| Submission Date: | 21-08-26 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to interface Real-Time Clock. Debug and verify the program using IDE on a development computer system.

#include <LPC21xx.h>

/* LCD Pins */

#define RS (1 << 16)

#define EN (1 << 17)

#define D4 (1 << 18)

#define D5 (1 << 19)

#define D6 (1 << 20)

#define D7 (1 << 21)

void delay(unsigned int count)

{

unsigned int i, j;

for(i = 0; i < count; i++)

{

for(j = 0; j < 1000; j++);

}

}

/* Send data to LCD */

void lcd_data(unsigned char data)

{

IOCLR0 = D4 | D5 | D6 | D7;

if(data & 0x10)

IOSET0 = D4;

if(data & 0x20)

IOSET0 = D5;

if(data & 0x40)

IOSET0 = D6;

if(data & 0x80)

IOSET0 = D7;

IOSET0 = RS;

IOSET0 = EN;

delay(1);

IOCLR0 = EN;

IOCLR0 = D4 | D5 | D6 | D7;

if(data & 0x01)

IOSET0 = D4;

if(data & 0x02)

IOSET0 = D5;

if(data & 0x04)

IOSET0 = D6;

if(data & 0x08)

IOSET0 = D7;

IOSET0 = EN;

delay(1);

IOCLR0 = EN;

delay(2);

}

/* Send command to LCD */

void lcd_cmd(unsigned char cmd)

{

IOCLR0 = RS;

IOCLR0 = D4 | D5 | D6 | D7;

if(cmd & 0x10)

IOSET0 = D4;

if(cmd & 0x20)

IOSET0 = D5;

if(cmd & 0x40)

IOSET0 = D6;

if(cmd & 0x80)

IOSET0 = D7;

IOSET0 = EN;

delay(1);

IOCLR0 = EN;

IOCLR0 = D4 | D5 | D6 | D7;

if(cmd & 0x01)

IOSET0 = D4;

if(cmd & 0x02)

IOSET0 = D5;

if(cmd & 0x04)

IOSET0 = D6;

if(cmd & 0x08)

IOSET0 = D7;

IOSET0 = EN;

delay(1);

IOCLR0 = EN;

delay(2);

}

void lcd_string(char *str)

{

while(*str)

{

lcd_data(*str);

str++;

}

}

void lcd_init(void)

{

IODIR0 |= RS | EN | D4 | D5 | D6 | D7;

delay(20);

lcd_cmd(0x02);

lcd_cmd(0x28);

lcd_cmd(0x0C);

lcd_cmd(0x06);

lcd_cmd(0x01);

delay(5);

}

void lcd_number(unsigned int num)

{

lcd_data((num / 10) + '0');

lcd_data((num % 10) + '0');

}

/* RTC Initialization */

void rtc_init(void)

{

CCR = 0x00;

PREINT = 0x000001C7;

PREFRAC = 0x000061C0;

SEC = 0;

MIN = 0;

HOUR = 12;

DOM = 19;

MONTH = 8;

YEAR = 2026;

CCR = 0x01;

}

int main(void)

{

PINSEL0 = 0x00000000;

PINSEL1 = 0x00000000;

lcd_init();

rtc_init();

while(1)

{

/* Display Time */

lcd_cmd(0x80);

lcd_string("TIME:");

lcd_number(HOUR);

lcd_data(':');

lcd_number(MIN);

lcd_data(':');

lcd_number(SEC);

/* Display Date */

lcd_cmd(0xC0);

lcd_string("DATE:");

lcd_number(DOM);

lcd_data('/');

lcd_number(MONTH);

lcd_data('/');

lcd_number(YEAR % 100);

delay(200);

}

}

OUTPUT: