DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #11

Activity Name: UART Programming

| Name of Student: | Agneay B Nair |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24102 |
| Activity Date: | 21-08-26 |
| Submission Date: | 21-08-26 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to drive UART module. Debug and verify the program using IDE on a development computer system.

CODE:

#include <LPC214x.h>

void UART0_Init(void);

void UART0_SendChar(char ch);

void UART0_SendString(char *str);

char UART0_ReceiveChar(void);

void UART0_Init(void)

{

// Select UART0 function

// P0.0 = TXD0

// P0.1 = RXD0

PINSEL0 = 0x00000005;

// Set Peripheral Clock = CCLK = 12 MHz

VPBDIV = 0x01;

// 8-bit data, 1 stop bit, no parity

// Enable DLAB to set baud rate

U0LCR = 0x83;

// Set baud rate = 9600

// PCLK = 12 MHz

U0DLL = 78;

U0DLM = 0;

// Disable DLAB

U0LCR = 0x03;

}

void UART0_SendChar(char ch)

{

// Wait until transmitter is ready

while (!(U0LSR & 0x20));

// Send character

U0THR = ch;

}

void UART0_SendString(char *str)

{

while (*str)

{

UART0_SendChar(*str);

str++;

}

}

char UART0_ReceiveChar(void)

{

// Wait until data is received

while (!(U0LSR & 0x01));

// Return received character

return U0RBR;

}

int main(void)

{

char data;

// Initialize UART0

UART0_Init();

// Initial message

UART0_SendString("UART Initialized\r\n");

UART0_SendString("Type a character: ");

while (1)

{

// Receive character

data = UART0_ReceiveChar();

// Display received character

UART0_SendString("\r\nReceived: ");

UART0_SendChar(data);

// Ask for another character

UART0_SendString("\r\nType a character: ");

}

}

OUTPUT: