DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #9

Activity Name: Timer Programming

| Name of Student: | Agneay B Nair |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24102 |
| Activity Date: | 21-08-26 |
| Submission Date: | 21-08-26 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to drive Timer module. Debug and verify the program using IDE on a development computer system.

CODE:

#include <LPC214x.h>

void timer0_delay(unsigned int delay_ms)

{

T0TCR = 0x02;          // Reset Timer0

T0PR = 14999;          // Prescaler for 1 ms delay

T0TC = 0;              // Clear Timer Counter

T0MR0 = delay_ms;      // Match value

T0MCR = 0x04;          // Stop timer when TC = MR0

T0TCR = 0x01;          // Start Timer0

while(!(T0IR & 0x01)); // Wait until match occurs

T0IR = 0x01;           // Clear interrupt flag

}

int main()

{

// Configure P0.10 as GPIO

PINSEL0 &= ~(3 << 20);

// Configure P0.10 as output

IO0DIR |= (1 << 10);

while(1)

{

IO0SET = (1 << 10);    // LED ON

timer0_delay(1000);    // 1 second delay

IO0CLR = (1 << 10);    // LED OFF

timer0_delay(1000);    // 1 second delay

}

}

OUTPUT: