DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #4

Activity Name: LCD Interfacing

| Name of Student: | Agneay B Nair |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24102 |
| Activity Date: | 18-08-26 |
| Submission Date: | 18-08-26 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to interface LCD. Debug and verify the program using IDE on a development computer system.

CODE:

#include <LPC213x.h>

#define RS (1 << 8)

#define EN (1 << 9)

void delay(unsigned int count)

{

unsigned int i, j;

for(i = 0; i < count; i++)

for(j = 0; j < 1000; j++);

}

void lcd_command(unsigned char cmd)

{

// Clear old data from P0.16 to P0.23

IOCLR0 = 0x00FF0000;

// Send command to P0.16-P0.23

IOSET0 = ((unsigned int)cmd << 16);

// RS = 0 for command

IOCLR0 = RS;

// Enable pulse

IOSET0 = EN;

delay(1);

IOCLR0 = EN;

delay(2);

}

void lcd_data(unsigned char data)

{

// Clear old data

IOCLR0 = 0x00FF0000;

// Send data to P0.16-P0.23

IOSET0 = ((unsigned int)data << 16);

// RS = 1 for data

IOSET0 = RS;

// Enable pulse

IOSET0 = EN;

delay(1);

IOCLR0 = EN;

delay(2);

}

void lcd_string(char *str)

{

while(*str != '\0')

{

lcd_data(*str);

str++;

}

}

void lcd_init(void)

{

delay(20);

lcd_command(0x38);   // 8-bit mode, 2 lines

lcd_command(0x0C);   // Display ON, cursor OFF

lcd_command(0x06);   // Auto increment cursor

lcd_command(0x01);   // Clear LCD

delay(2);

}

int main(void)

{

// Configure pins as GPIO

PINSEL0 = 0x00000000;

PINSEL1 = 0x00000000;

// P0.8, P0.9 and P0.16-P0.23 as output

IODIR0 = 0x00FF0300;

lcd_init();

// First line

lcd_command(0x80);

lcd_string("Hello!");

// Second line

lcd_command(0xC0);

lcd_string("LPC2138 LCD");

while(1);

}

OUTPUT: