DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #6

Activity Name: ADC Module Interfacing

| Name of Student: | Agneay B Nair |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24102 |
| Activity Date: | 18-08-26 |
| Submission Date: | 18-08-26 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to drive ADC module. Debug and verify the program using IDE on a development computer system.

CODE:

#include <LPC213x.h>

#define RS (1 << 16)

#define EN (1 << 17)

unsigned int adc_value;

void delay(unsigned int count)

{

unsigned int i, j;

for(i = 0; i < count; i++)

for(j = 0; j < 100; j++);

}

/* Send command to LCD */

void lcd_cmd(unsigned char cmd)

{

IOCLR0 = RS;

/* Send higher nibble */

IOCLR0 = 0x003C0000;       // Clear P0.18 to P0.21

IOSET0 = ((cmd & 0xF0) << 14);

IOSET0 = EN;

delay(10);

IOCLR0 = EN;

/* Send lower nibble */

IOCLR0 = 0x003C0000;

IOSET0 = ((cmd & 0x0F) << 18);

IOSET0 = EN;

delay(10);

IOCLR0 = EN;

}

/* Send data to LCD */

void lcd_data(unsigned char data)

{

IOSET0 = RS;

/* Send higher nibble */

IOCLR0 = 0x003C0000;

IOSET0 = ((data & 0xF0) << 14);

IOSET0 = EN;

delay(10);

IOCLR0 = EN;

/* Send lower nibble */

IOCLR0 = 0x003C0000;

IOSET0 = ((data & 0x0F) << 18);

IOSET0 = EN;

delay(10);

IOCLR0 = EN;

}

/* Display string */

void lcd_string(char *str)

{

while(*str)

{

lcd_data(*str);

str++;

}

}

/* Display ADC number */

void lcd_number(unsigned int num)

{

unsigned char thousands, hundreds, tens, ones;

thousands = num / 1000;

num = num % 1000;

hundreds = num / 100;

num = num % 100;

tens = num / 10;

ones = num % 10;

if(thousands)

lcd_data(thousands + '0');

if(thousands || hundreds)

lcd_data(hundreds + '0');

if(thousands || hundreds || tens)

lcd_data(tens + '0');

lcd_data(ones + '0');

}

/* Initialize LCD */

void lcd_init(void)

{

delay(100);

lcd_cmd(0x02);     // 4-bit mode

lcd_cmd(0x28);     // 2 lines, 5x7 matrix

lcd_cmd(0x0C);     // Display ON, cursor OFF

lcd_cmd(0x06);     // Increment cursor

lcd_cmd(0x01);     // Clear display

delay(100);

}

int main(void)

{

/* LCD pins P0.16 to P0.21 as GPIO */

PINSEL1 = 0x01000000;

/* P0.16 to P0.21 as output */

IODIR0 |= 0x003F0000;

lcd_init();

lcd_cmd(0x80);

lcd_string("ADC VALUE:");

while(1)

{

/* Configure and enable ADC0 Channel 1 */

AD0CR = (1 << 1) | (10 << 8) | (1 << 21);

/* Start ADC conversion */

AD0CR |= (1 << 24);

/* Wait for conversion */

while(!(AD0GDR & (1UL << 31)));

/* Extract 10-bit ADC value */

adc_value = (AD0GDR >> 6) & 0x3FF;

/* Move to second line */

lcd_cmd(0xC0);

/* Display ADC value */

lcd_number(adc_value);

delay(500);

}

}

OUTPUT: